the code of version 5004 is more credible ,POST card self-test is more detailed

# User's guide of four-bit code POST card

This User's guide is fit for the common computer post card (the PI0050) and the note book computer post card (M04).

四位代码形电脑主板故障诊断卡使用说明

（There are editions in Chinese for you to choose）

可选配中文说明书

● Not only diagnose the trouble of main board but also the trouble of the POST card, keep users from misunderstanding .

● More compatible with main board PIV, end the history that the POST card is unable to work as the main board update constantly.

● You can consult the code that has run; Press the switch once and the code pauses

● It do no harms to device while insert the card wrongly with a speaker on the card to remind you there is an error, SMD device that protect your hands.

- Test the speed of PCI and ISA bus of the computer, you will get the result as soon as you test it. It can help you not suffer losses when you buy the computer, and also help you sell the computer that has a fast bus speed at good price.  .

Catalogue

I、Front View                                                  2

II、Implication of word/number of four-bit code POST card               3

III、Flow chart                                                 4

Ⅳ、Distinguish true and false                                          7

Ⅴ、OBLIGATORY CONTENT                                   7

Ⅵ、Description of LED displays                                  8

Ⅶ、Error code table                                             8

Ⅷ、Description of beep code                                     30

Ⅸ、 Frequently-asked questions                                 34

- Front View

This card that named PI0050 is fit for common computer

II、Implication of word/number of four-bit code post card

This card named M04 is fit for notebook computer.

| word/NUM | Description |
| --- | --- |
| ０－－－ | Start automatic diagnosis after it displayed for about half a second |
| 1－－－ | The first function symbol of main menu ,then enter code consulting function after it displayed for about half a second |
| 2－－－ | The second function symbol of main menu , then display the reference speed of bus after it displayed for about half a second |
| 3－－－ | The third function symbol of main menu ,display the versionnumber"5002" after it displayed for about half a second |
| 4－－－ | The fourth function symbol of main menu, start to test the card and display from "0000"、"1111" to "FFFF" after it displayed for about half a second |
| 5－－－ | The 5th function prompt ,start self-test after the prompt displayed for about half second. .As long as each of 4 bits can display symbols, no matter what it displays, the POST process passed. Because of the POST content has been enhanced a lot, plenty of symbols are especial . you can pay no attention to it. |
| －P C I | It indicates that the slot you insert the card is PCI slot, and wait for you to consult the next code by pressing function switch. |
| －I S A | It indicates that the slot that you insert the card is ISA slot. And wait for you to consult the next code by pressing function switch. |
| －－－Ｐ | Waiting for you to consult the next code by pressing the function switch. After it displayed for half a second, the code will be displayed, and the first two bits indicates the hexadecimal error code .The last two bits indicates the ordinal number of the code. |
| Ｐ－－－ | Waiting for you to consult the preceding code by pressing the function switch. After it displayed for half a second, the code will be displayed, and the first two bits indicates the hexadecimal error code, the last two bits indicates the ordinal number of the code. |
| －E n d | The last code( ordinal number limit: 0-47) forward consulting is displayed; press and hold the function switch for some 0.8 second, then enter the backward consulting mode and display "P- - -", after half a second, the code is displayed, the first two-bit code indicates the 47th POST code, the last two-bit code(47) indicates the ordinal number of the code; if press and hold the function switch for about 0.8 second twice, it will exit the code consulting and enter the second function of main menu, at the same time displayed "2- - -", then display the reference speed of PCI/ISA bus in decimal in half a second. |
| E n d－ | The first code (ordinal number limit : 0-47) backward consulting is displayed; Press and hold the function switch for 0.8 second, then enter the forward consulting mode and display"- - -P", after half a second, the code is displayed, the first two-bit code indicates the zero code; the last two-bit code (00) indicates the ordinal number of the code. If press and hold the function switch for about 0.8 second twice, it will exit the code consulting and enter the second function of main menu, at the same time displayed"2- - - ", then display the reference speed of PCI/ISA bus in decimal in half a second. |

III. Flow chart

Begin

Power off and remove all the cards that plug in expansion slot. Insert the card into ISA or PCI slot. (Notice: When you plug it in the ISA slot the component side should face to the power, if it plugged in the wrong direction, the card and motherboard is not broken, but both the card and the main board stop running.

According to the “Description of LED displaying ”, you can find the cause of the trouble and correct it

Power on; Judge If the all lights run normally. (The BIOS light may be on low power to sparkle

NO

YES

According to the error code table, you can find the cause of the trouble and correct it

YES

Judge whether the error code that means there is a trouble with the main board is displayed

NO

Power off, insert the display card、I/O card、keyboard、hard disk drive and expansion cards

Power on, Judge whether the error code that means there is a trouble with the main board is displayed

Power off, According to the error code table, you can find the cause of the trouble and correct it

YES

NO

End

If the result of the test is correct even though it can’t boot the operating system, there may be some trouble with the software or disc drive, or disk controller, or DMA circuit.

Illustration of four-bit code POST card (5004 version)

power on

Display "0- - - "

testing...

Error   Preceding

Code   Code

8  8   8  8

Display "- PCI" if insert to the PCI slot, Display "- ISA" if insert to the ISA slot,

Press and hold the

switch for about

0.8 second

Display "1- - - " about 1 second   latter code consulting

Display reference speed of bus in decimal

Display "2- - - "

test speed

Press and hold the switch for about 0.8 second

Display "3- - - "

display version

number

Display version number

5  0  0   4

If the display component is ok, it will display from"0000.1111.2222.3333"to"FFFF"

Press and hold the switch for about 0.8 second

Display "4- - - "

self-test display

component

F   F   F   F

As long as each of 4 bits can display symbols, no matter what it displays, the  POST process passed. Because of the POST content has be enhanced a lot, plenty of symbols are especial. you can pay no attention to it.

8  8  8   8

Press and hold the switch for about 0.8 second

Display "5- - - "

RAM、ROM

self-test (The display is so fast that you cannot see.)

Connecting dot Ⅱ

Connecting dot Ⅰ

Connecting dot Ⅰ

press and hold for about 0.8 second

Connecting dot Ⅱ

display "- - - P" consulted the forward

press and hold for about 0.8 second

display "End - " , the first code of 48 codes

press and hold the switch for about 0.4 second

(the 49th)

POST　ordinal　number

Code　　of the code

press and hold the

switch for about 0.4

second (the 2nd)

８ ８　  ０ ０

press and hold the switch for

about 0.4 second

press and hold the

switch for about 0.4

second (the 2nd)

POST　ordinal　number

Code　　of the code

8   8     0  1

press and hold the

switch for about 0.4second

(the 47th)

press and hold the

switch for about 0.4

second (the 2nd)

POST　   MAX

Code

press and hold the

switch for about 0.4second

(the 1st)

8  0     4   7

press and hold the

switch for about 0.4second

(the 49th)

display "- End" the last code

press and hold the switch for about 0.8 second

display "P- - - "  consult it backward

press and hold the switch for about 0.8 second

exit the code consulting

code analysis clew

The codes that can be consulted are last 0 to 47 codes that take relative longer time in the POST process, not all the code need to be consulted.(some main board output millions of codes or the same code appears many times), If the same code take different time to run in different running courses on the same main board , the POST card not always use it as code that can be consulted. The main board is of the good stability If the 47 codes that consulted in different running courses on the same main board are the same, and the last  47th code has passed the POST, but this situation is rare.

Ⅳ、Distinguish true and false

Pi0050is more suitable than Pi0049for slap-up main board ,such as pill、PIV and so on. So you can distinguish them by their characteristics ,and also you can dial this number 086 139 2517 4332 or write to me by E-mail:p678@163.net to get lastest   distinguishing message.

TRUE

●There are typefaces like “China Copyright 01224987.4”on the edge of PCB;

●There are typefaces like “China Copyright 513427”on PI0049、pi0050;

FLASE

●On the back of the card ,there is a telephone number written like “086 139 2517 4332;

Known characteristic of spurious cards:

●”中国专有号：01223987.3”

●”专有技术：01224988.3”

●”专利号：02125087.5”(It is the patent of biology organic compound)

●Be careful .The spurious cards always use badly or unsuitable materials, it hasn’t been tested by the professional equipment ,and has no simulation technique.

- At the same time ,there may be a few low card appears to used as PI0050 card.

Ⅴ、OBLIGATORY CONTENT

1. The error code table is in the order of the code value that from small to big. The sequence that the code displays is decided by BIOS of the motherboard;

2. Four-bit code can be divided to two two-bit codes .The one is made of the thousands digit and the hundreds digit ;The other is made of the tens digit and units digit .According to the two two-bit codes, Not only you can know the posting for computer can not pass the units that the thousands digit and the hundreds digit point out, but also you can know that the post for computer can pass the units at last that the tens digit and the units digit point out;

3. Code haven’t be defined is not included in the table;

4. For the different BIOS (such as AMI、Award、Phoenix ), a Code has different

meanings. So make sure that which kind of BIOS you are testing. Or view the user’s guide, or See it on the BIOS IC on the motherboard;

5. There is only some code displayed when you insert the card into the PCI slot on a few motherboards, but when it plugged into the ISA slot, all the code could be displayed. At present, it has be discovered that the code is displayed when you insert the card into the PCI slot of several computers which has registered trade mark, but not ISA. So You’d better try it on the other slot if the code is not displayed. In addition, on the different PCI slots of a board, some could display the code, for example, the code is displayed and goes from “00” to “FF” when you insert the card into the PCI slot, which is near to the CPU on motherboard DELL810, but if in the other slot , the code would stopped at the port “38”;

6.  The time that reset message output needed is not always in-phase, so sometimes the code is displayed when the card in the ISA, but it is stopped at the origination code when in the PCI.;

7. As there are more and more kinds motherboard, and the code of BIOS POST is updated ceaselessly, so the meanings of error codes is just for reference;

Ⅵ、Description of LED displays

| LED | Type | Description |
| --- | --- | --- |
| CLK | Bus clock | Lights when the power is applied to the empty board (even without CPU ) , or else there is no message. |
| BIOS | Base input/output read | LED that turn on and off when the board is powered on, as CPU is reading to BIOS. |
| IRDY | Manager is ready | LED that turn on and off when there is a message. |
| OSC | Oscillation | Lights when the board is powered on, or else the crystal oscillation circuit is broken, and has no OSC message. |
| FRAME | Frame periods | Lights all the time. Turn on and off only when there is a circular frame message. |
| RST | Reset | Lights only for half-second when you slide the power switch or the reset switch. If it is lit all the time, check the following: make sure that the reset pin is plugged properly, or the reset circuit is broken. |
| 12V | Power | Lights once the board is powered on, if it is not lit, that means the short circuit occurs on motherboard, or voltage can’t up to 12V. |
| -12V | Power | The same as“12V” |
| 5V | Power | The same as “12V” |
| -5V | Power | The same as “12V” (-5V is output only in ISA slot. |

Ⅶ、Error code table

| CODE |  | Award | AMI | Phoenix4.0/Tandy3000 |
| --- | --- | --- | --- | --- |
| CODE |  | Award | AMI | Phoenix4.0/Tandy3000 |
| 00 |  |  | Code copying to specific areas is done. Passing control to INT 19h boot loader next. |  |
| 01 |  | Processor Test 1, Processor status (1FLAGS) verification. Test the following processor status flags: carry, zero, sign, overflow. |  | CPU is testing the register inside or failed, please change the CPU and check it. |
|  |  | The BIOS sets each flag, verifies they are set, then turns each flag off and verifies it is off. |  |  |
| 02 |  | Test All CPU Registers Except SS, SP, and BP with Data FF and 00 |  | Verify Real Mode |
| 03 |  | Disable NMI, PIE, AIE, UEI, SQWV. | The NMI is disabled. Next, checking for a soft reset or a power on condition | Disable Not masked Interrupt (NMI) |
|  |  | Disable video, parity checking, DMA. |  |  |
|  |  | Reset math coprocessor. |  |  |
|  |  | Clear all page registers, CMOS shutdown byte. |  |  |
|  |  | Initialize timer 0, 1, and2, including set EISA timer to a known state. |  |  |
|  |  | Initialize DMA controllers 0 and 1. |  |  |
|  |  | Initialize interrupt controllers 0 and 1. |  |  |
|  |  | Initialize EISA extended registers. |  |  |
| 04 |  | RAM must be periodically refreshed to keep the memory from decaying. This refresh function is working properly. |  | Get CPU type |
| 05 |  | Keyboard Controller Initialization | The BIOS stack has been built. Next, disabling cache memory. | DMA initialization in progress or failure |
| CODE |  | Award | AMI | Phoenix4.0/Tandy3000 |
| 06 |  | Reserved | Uncompressing the POST code next. | Initialize system hardware |
| 07 |  | Verifies CMOS is Working Correctly, Detects Bad Battery | Next, initializing the CPU and the CPU data area | Disable shadow and execute code from the ROM. |
| 08 |  | Early chip set initialization | The CMOS checksum calculation is | Initialize chipset with initial POST values |
|  |  | Memory presence test |  |  |
|  |  | OEM chip set routines |  |  |
|  |  | Clear low 64K memory |  |  |
|  |  | Test first 64K memory |  |  |
| 09 |  | Cyrix CPU initialization |  | Set IN POST flag |
|  |  | Cache initialization |  |  |
| 0A |  | Initialize first 120 interrupt vectors with SPURIOUS-INT-HDLR and initialize INT 00h-1Fh according to INT-TBL. | The CMOS checksum calculation is done. Initializing the CMOS status register for date and time next. | Initialize CPU registers |
| 0B |  | Test CMOS RAM Checksum, if Bad, or INS Key Pressed, Load Defaults | The CMOS status register is initialized. Next, performing any required initialization before the keyboard BAT command is issued | Enable CPU cache |
| 0C |  | Detect Type of Keyboard Controller and | The keyboard controller input buffer is free. Next, issuing the BAT command to the keyboard controller. | Initialize caches to initial POST values |
|  |  | Set NUM_LOCK Status |  |  |
| 0D |  | Detect CPU Clock; |  |  |
|  |  | Read CMOS location 14h to find out type of video in use. |  |  |
|  |  | Detect and initialize video adapter. |  |  |
| CODE |  | Award | AMI | Phoenix4.0/Tandy3000 |
| 0E |  | Test Video Memory, write sign-on message to screen. | The keyboard controller BAT command result has been verified. Next, performing any necessary initialization after the keyboard controller BAT command test | Initialize I/O component |
|  |  | Setup shadow RAM ?Enable shadow according to setup. |  |  |
| 0F |  | Test DMA Cont. 0; BIOS Checksum Test. | The initialization after the keyboard controller BAT command test is done. The keyboard command byte is written next. | Initialize the local bus IDE |
|  |  | Keyboard Detect and Initialization. |  |  |
| 10 |  | Test DMA Controller 1 | The keyboard controller command byte is written. Next, issuing the Pin 23 and 24 blocking and unblocking command | Initialize Power Management |
| 11 |  | Test DMA Page Registers | Next, checking if <End> or <Ins> keys were pressed during power on. Initializing CMOS RAM if the Initialize CMOS RAM in every boot AMIBIOS POST option was set in AMIBCP or the <End> key was pressed. | Load alternate registers with initial POST values |
| 12 |  | Reserved | Next, disabling DMA controllers 1 and 2 and interrupt controllers 1 and 2 | Restore CPU control word during warm boot |
| 13 |  | Reserved | The video display has been disabled. Port B has been initialized. Next, initializing the chipset | Initialize PCI Bus Mastering devices |
| CODE |  | Award | AMI | Phoenix4.0/Tandy3000 |
| 14 |  | Test 8254 Timer 0 Counter 2 | The 8254 timer test will begin next. | Initialize keyboard controller |
| 15 |  | Verify 8259 Channel 1 Interrupts by Turning Off and On the Interrupt Lines |  |  |
| 16 |  | Verify 8259 Channel 2 Interrupts by Turning Off and On the Interrupt Lines |  | BIOS ROM checksum |
| 17 |  | Turn Off Interrupts Then Verify No Interrupt Mask Register is On |  | Initialize cache before memory Auto size |
| 18 |  | Force an Interrupt and Verify the Interrupt Occurred |  | 8254 timer initialization |
| 19 |  | Test Stuck NMI Bits; Verify NMI Can Be Cleared | The 8254 timer test is over. Starting the memory refresh test next |  |
| 1A |  | Display CPU clock | The memory refresh line is toggling. Checking the 15 second on/off time next | 8237 DMA controller initialization |
| 1B |  | reserved |  |  |
| 1C |  | Reserved |  | Reset Programmable Interrupt Controller |
| 1D |  | Reserved |  |  |
| 1E |  | Reserved |  |  |
| 1F |  | If EISA non-volatile memory checksum is good, execute EISA initialization. |  |  |
|  |  | If not, execute ISA tests an clear. |  |  |
|  |  | EISA mode flag. |  |  |
|  |  | Test EISA configuration memory |  |  |
|  |  | Integrity (checksum & communication interface). |  |  |
| 20 |  | Initialize Slot 0 (System Board) |  | Test DRAM refresh |
| 21 |  | Initialize Slot 1 |  |  |
| 22 |  | Initialize Slot 2 |  | Test 8742 Keyboard Controller |
| CODE |  | Award | AMI | Phoenix4.0/Tandy3000 |
| 23 |  | Initialize Slot 3 | Reading the 8042 input port and disabling the MEGAKEY Green PC feature next. Making the BIOS code segment rewritable and performing any necessary configuration before initializing the interrupt vectors |  |
| 24 |  | Initialize Slot 4 | The configuration required before interrupt vector initialization has completed. Interrupt vector initialization is about to begin | Set ES segment register to 4 GB |
| 25 |  | Initialize Slot 5 | Interrupt vector initialization is done. Clearing the password if the POST DIAG switch is on. |  |
| 26 |  | Initialize Slot 6 |  |  |
| 27 |  | Initialize Slot 7 | Any initialization before setting video mode will be done next |  |
| 28 |  | Initialize Slot 8 | Initialization before setting the video mode is complete. Configuring the monochrome mode and color mode settings next | Auto size DRAM |
| 29 |  | Initialize Slot 9 |  | Initialize POST Memory Manager |
| 2A |  | Initialize Slot 10 | Initializing the different bus system, static, and output devices, if present | Clear 512 KB base RAM |
| CODE |  | Award | AMI | Phoenix4.0/Tandy3000 |
| 2B |  | Initialize Slot 11 | Passing control to the video ROM to perform any required configuration before the video ROM test. |  |
| 2C |  | Initialize Slot 12 | All necessary processing before passing control to the video ROM is done. Looking for the video ROM next and passing control to it. | RAM failure on address line XXXX* |
| 2D |  | Initialize Slot 13 | The video ROM has returned control to BIOS POST. Performing any required processing after the video ROM had control |  |
| 2E |  | Initialize Slot 14 | Completed post-video ROM test processing. If the EGA/VGA controller is not found, performing the display memory read/write test next | RAM failure on data bits XXXX* of low byte of memory bus |
| 2F |  | Initialize Slot 15 | The EGA/VGA controller was not found. The display memory read/write test is about to begin | Enable cache before system BIOS shadow |
| 30 |  | Size Base Memory From 256K to 640K and Extended Memory Above 1MB | The display memory read/write test passed. Look for retrace checking next |  |
| 31 |  | Test Base Memory From 256K to 640K and Extended Memory Above 1MB | The display memory read/write test or retrace checking failed. Performing the alternate display memory read/write test next |  |
| CODE |  | Award | AMI | Phoenix4.0/Tandy3000 |
| 32 |  | If EISA Mode, Test EISA Memory Found in Slots Initialization | The alternate display memory read/write test passed. Looking for alternate display retrace checking next. | Test CPU bus-clock frequency |
| 33 |  | Reserved |  | Initialize Phoenix Dispatch manager |
| 34 |  | Reserved | Video display checking is over. Setting the display mode next. |  |
| 35 |  | Reserved |  |  |
| 36 |  | Reserved |  | Warm start and shut down |
| 37 |  | Reserved | The display mode is set. Displaying the power on message next |  |
| 38 |  | Reserved | Initializing the bus input, IPL, general devices next, if present | Shadow system BIOS ROM |
| 39 |  | Reserved | Displaying bus initialization error messages. |  |
| 3A |  | Reserved | The new cursor position has been read and saved. Displaying the Hit <DEL> message next | Auto size cache |
| 3B |  | Reserved | The Hit <DEL> message is displayed. The protected mode memory test is about to start. |  |
| 3C |  | Setup Enabled |  | Advanced configuration of chipset registers |
| 3D |  | Detect if Mouse is Present, Initialize Mouse, Install Interrupt Vectors |  | Load alternate registers with CMOS values |
| 3E |  | Initialize Cache Controller |  |  |
| 3F |  | Reserved |  |  |
| 40 |  | Display Virus Protest Disable or Enable | Preparing the descriptor tables next |  |
| 41 |  | Initialize Floppy Disk Drive Controller and Any Drives |  | Initialize extended memory for Rom Pilot |
| CODE |  | Award | AMI | Phoenix4.0/Tandy3000 |
| 42 |  | Initialize Hard Drive Controller and Any Drives | The descriptor tables are prepared. Entering protected mode for the memory test next | Initialize interrupt vectors |
| 43 |  | Detect and Initialize Serial & Parallel Ports and Game Port | Entered protected mode. Enabling interrupts for diagnostics mode next. |  |
| 44 |  | Reserved | Interrupts enabled if the diagnostics switch is on. Initializing data to check memory wraparound at 0:0 next. |  |
| 45 |  | Detect and Initialize Math Coprocessor | Data initialized. Checking for memory wraparound at 0:0 and finding the total system memory size next | POST device initialization |
| 46 |  | Reserved | The memory wraparound test is done. Memory size calculation has been done. Writing patterns to test memory next | Check ROM copyright notice |
| 47 |  | Reserved | The memory pattern has been written to extended memory. Writing patterns to the base 640 KB memory next. | Initialize I20 support |
| 48 |  | Reserved | Patterns written in base memory. Determining the amount of memory below 1 MB next. | Check video configuration against CMOS |
| 49 |  | Reserved | The amount of memory below 1 MB has been found and verified. Determining the amount of memory above 1 MB memory next. | Initialize PCI bus and devices |
| CODE |  | Award | AMI | Phoenix4.0/Tandy3000 |
| 4A |  | Reserved |  | Initialize all video adapters in system |
| 4B | Reserved |  | The amount of memory above 1 MB has been found and verified. Checking for a soft reset and clearing the memory below 1 MB for the soft reset next. If this is a power on situation, going to checkpoint 4Eh next. | Quiet Boot start (optional) |
| 4C | Reserved |  | The memory below 1 MB has been cleared via a soft reset. Clearing the memory above 1 MB next. | Shadow video BIOS ROM |
| 4D | Reserved |  | The memory above 1 MB has been cleared via a soft reset. Saving the memory size next. Going to checkpoint 52h next |  |
| 4E | Reboot if Manufacturing Mode; If not, Display Messages and Enter Setup |  | The memory test started, but not as the result of a soft reset. Displaying the first 64 KB memory size next. | Display BIOS copyright notice |
| 4F | Ask Password Security (Optional) |  | The memory size display has started. The display is updated during the memory test. Performing the sequential and random memory test next | Initialize Multi Boot |
| 50 | Write All CMOS Values Back to RAM and Clear |  | The memory below 1 MB has been tested and initialized. Adjusting the displayed memory size for relocation and shadowing next. | Display CPU type and speed |
| CODE | Award |  | AMI | Phoenix4.0/Tandy3000 |
| 51 | Enable Parity Checker. Enable NMI, Enable Cache Before Boot |  | The memory size display was adjusted for relocation and shadowing. Testing the memory above 1 MB next. | Initialize EISA board |
| 52 | Initialize Option ROMs from C8000h to EFFFFh or if FSCAN Enabled to F7FFFh |  | The memory above 1 MB has been tested and initialized. Saving the memory size information next. | Test keyboard |
| 53 | Initialize Time Value in 40h: BIOS Area |  | The memory size information and the CPU registers are saved. Entering real mode next. |  |
| 54 |  |  | Shutdown was successful. The CPU is in real mode. Disabling the Gate A20 line, parity, and the NMI next | Set key click if enabled |
| 55 |  |  |  | Enable USB devices |
| 57 |  |  | The A20 address line, parity, and the NMI are disabled. Adjusting the memory size depending on relocation and shadowing next. |  |
| 58 |  |  | The memory size was adjusted for relocation and shadowing. Clearing the Hit <DEL> message next | Test for unexpected interrupts |
| 59 |  |  | The Hit <DEL> message is cleared. The <WAIT...> message is displayed. Starting the DMA and interrupt controller test next. | Initialize POST display service |
| 5A |  |  |  | Display prompt 'Press F2 to enter SETUP' |
| CODE | Award |  | AMI | Phoenix4.0/Tandy3000 |
| 5B |  |  |  | Disable CPU cache |
| 5C |  |  |  | Test RAM between 512 and 640 KB |
| 60 | Setup virus protection (boot sector protection) functionality according to setup setting. |  | The DMA page register test passed. Performing the DMA Controller 1 base register test next. | Test extended memory |
| 61 | Try to turn on level 2 cache (if L2 cache already turned on in post 3D, this part will be skipped) |  |  |  |
|  | Set the boot up speed according to setup setting |  |  |  |
|  | Last chance for chipset initialization |  |  |  |
|  | Last chance for power management initialization (Green BIOS only) |  |  |  |
|  | Show the system configuration table |  |  |  |
| 62 | Setup NUM Lock Status According to Setup values |  | The DMA controller 1 base register test passed. Performing the DMA controller 2 base register test next | Test extended memory address lines |
|  | Program the NUM lock, typematic rate & typematic speed according to setup setting |  |  |  |
| 63 | If there is any changes in the hardware configuration, update the ESCD information (PnP BIOS only) |  |  |  |
|  | Clear memory that have been used |  |  |  |
|  | Boot system via INT 19h |  |  |  |
| 64 |  |  |  | Jump to UserPatch1 |
| 65 |  |  | The DMA controller 2 base register test passed. Programming DMA controllers 1 and 2 next |  |
| CODE | Award |  | AMI | Phoenix4.0/Tandy3000 |
| 66 |  |  | Completed programming DMA controllers 1 and 2. Initializing the 8259 interrupt controller next. | Configure advanced cache registers |
| 67 |  |  | Completed 8259 interrupt controller initialization | Initialize Multi Processor APIC |
| 68 |  |  |  | Enable external and CPU caches |
| 69 |  |  |  | Setup System Management Mode (SMM) area |
| 6A |  |  |  | Display external L2 cache size |
| 6B |  |  |  | Load custom defaults (optional) |
| 6C |  |  |  | Display shadow-area message |
| 6E |  |  |  | Display possible high address for UMB recovery |
| 6F |  |  |  |  |
| 70 |  |  |  | Display error message |
| 71 |  |  |  |  |
| 72 |  |  |  | Check for configuration errors |
| 76 |  |  |  | Check for keyboard errors |
| 7C |  |  |  | Set up hardware interrupt vectors |
| 7D |  |  |  | Initialize Intelligent System Monitoring |
| 7E |  |  |  | Initialize coprocessor if present |
| 7F |  |  | Extended NMI source enabling is in progress. |  |
| CODE | Award |  | AMI | Phoenix4.0/Tandy3000 |
| 80 |  |  | The keyboard test has started. Clearing the output buffer and checking for stuck keys. Issuing the keyboard reset command next | Disable onboard Super I/O ports and IRQs |
| 81 |  |  | A keyboard reset error or stuck key was found. Issuing the keyboard controller interface test command next | Late POST device initialization |
| 82 |  |  | The keyboard controller interface test completed. Writing the command byte and initializing the circular buffer next. | Detect and install external RS232 ports |
| 83 |  |  | The command byte was written and global data initialization has completed. Checking for a locked key next | Configure non-MCD IDE controllers |
| 84 |  |  | Locked key checking is over. Checking for a memory size mismatch with CMOS RAM data next | Detect and install external parallel ports |
| 85 |  |  | The memory size check is done. Displaying a soft error and checking for a password or bypassing WINBIOS Setup next. | Initialize PC-compatible PnP ISA devices |
| 86 |  |  | The password was checked. Performing any required programming before WINBIOS Setup next | Re-initialize onboard I/O ports. |
| CODE | Award |  | AMI | Phoenix4.0/Tandy3000 |
| 87 |  |  | The programming before WINBIOS Setup has completed. Uncompressing the WINBIOS Setup code and executing the AMIBIOS Setup or WINBIOS Setup utility next | Configure Motherboard Configurable Devices (optional) |
| 88 |  |  | Returned from WINBIOS Setup and cleared the screen. Performing any necessary programming after WINBIOS Setup next | Initialize BIOS Data Area |
| 89 |  |  | The programming after WINBIOS Setup has completed. Displaying the power on screen message next | Enable Non-Maskable Interrupts (NMIs) |
| 8A |  |  |  | Initialize Extended BIOS Data Area |
| 8B |  |  | The first screen message has been displayed. The <WAIT...> message is displayed. Performing the PS/2 mouse check and extended BIOS data area allocation check next | Test and initialize PS/2 mouse |
| 8C |  |  | Programming the WINBIOS Setup options next | Initialize floppy controller |
| 8D |  |  | The WINBIOS Setup options are programmed. Resetting the hard disk controller next |  |
| CODE | Award |  | AMI | Phoenix4.0/Tandy3000 |
| 8E |  |  | The hard disk controller has been reset. Configuring the floppy drive controller next |  |
| 8F |  |  |  | Determine number of ATA drives (optional) |
| 90 |  |  |  | Initialize hard-disk controllers |
| 91 |  |  | The floppy drive controller has been configured. Configuring the hard disk drive controller next. | Initialize local-bus hard-disk controllers |
| 92 |  |  |  | Jump to UserPatch2 |
| 93 |  |  |  | Build MPTABLE for multi-processor boards |
| 95 |  |  | Initializing bus adaptor ROMs from C8000h through D8000h | Install CD ROM for boot |
| 96 |  |  | Initializing before passing control to the adaptor ROM at C800 | Clear huge ES segment register |
| 97 |  |  | Initialization before the C800 adaptor ROM gains control has completed. The adaptor ROM check is next. | Fix up Multi Processor table |
| 98 |  |  | The adaptor ROM had control and has now returned control to BIOS POST. Performing any required processing after the option ROM returned control A | Search for option ROMs. One long, two short beeps on checksum failure |
| CODE | Award |  | AMI | Phoenix4.0/Tandy3000 |
| 99 |  |  | Any initialization required after the option ROM test has completed. Configuring the timer data area and printer base address next. | Check for SMART Drive (optional) |
| 9A |  |  | Set the timer and printer base addresses. Setting the RS-232 base address next. | Shadow option ROMs |
| 9B |  |  | Returned after setting the RS-232 base address. Performing any required initialization before the Coprocessor test next. |  |
| 9C |  |  | Required initialization before the Coprocessor test is over. Initializing the Coprocessor next | Set up Power Management |
| 9D |  |  | Coprocessor initialized. Performing any required initialization after the Coprocessor test next. | Initialize security engine (optional) |
| 9E |  |  | Initialization after the Coprocessor test is complete. Checking the extended keyboard, keyboard ID, and Num Lock key next. Issuing the keyboard ID command next | Enable hardware interrupts |
| 9F |  |  |  | Determine number of ATA and SCSI drives |
| A0 |  |  |  | Set time of day |
| A1 |  |  |  | Check key lock |
| A2 |  |  | Displaying any soft error next |  |
| CODE | Award |  | AMI | Phoenix4.0/Tandy3000 |
| A3 |  |  | The soft error display has completed. Setting the keyboard typematic rate next. |  |
| A4 |  |  | The keyboard typematic rate is set. Programming the memory wait states next | Initialize typematic rate |
| A5 |  |  | Memory wait state programming is over. Clearing the screen and enabling parity and the NMI next |  |
| A7 |  |  | NMI and parity enabled. Performing any initialization required before passing control to the adaptor ROM at E000 next. |  |
| A8 |  |  | Initialization before passing control to the adaptor ROM at E000h completed. Passing control to the adaptor ROM at E000h next | Erase F2 prompt |
| A9 |  |  | Returned from adaptor ROM at E000h control. Performing any initialization required after the E000 option ROM had control next |  |
| AA |  |  | Initialization after E000 option ROM control has completed. Displaying the system configuration next | Scan for F2 key stroke |
| AB |  |  | Uncompressing the DMI data and executing DMI POST initialization next |  |
| AC |  |  |  | Enter SETUP |
| AE |  |  |  | Clear boot flag |
| CODE | Award |  | AMI | Phoenix4.0/Tandy3000 |
| B0 | If Interrupts Occurs in Protected Mode |  | The system configuration is displayed. | Check for errors |
| B1 | If Unmasked NMI Occurs, Display Press F1 to Disable NMI, F2 Reboot |  | Copying any code to specific areas. | Inform RomPilot about the end of POST. |
| B2 |  |  |  | POST done - prepare to boot operating system |
| B3 |  |  |  |  |
| B4 |  |  |  | 1 One short beep before boot |
| B5 |  |  |  | Terminate QuietBoot (optional |
| B6 |  |  |  | Check password (optional) |
| B7 |  |  |  | Initialize ACPI BIOS |
| B8 |  |  |  |  |
| B9 |  |  |  | Prepare Boot |
| BA |  |  |  | Initialize SMBIOS |
| BB |  |  |  | Initialize PnP Option ROMs |
| BC |  |  |  | Clear parity checkers |
| BD |  |  |  | Display MultiBoot menu |
| BE | Program chipset registers with power on BIOS defaults |  |  | Clear screen (optional) |
| BF | Program the rest of the chipset’s value according to setup (later setup value program) |  |  | Check virus and backup reminders |
|  | If auto configuration is enabled, programmed the chipset with predefined values in the MODBINable Auto Table |  |  |  |
| C0 | Turn off OEM specific cache, shadow |  |  | Try to boot with INT 19 |
|  | Initialize standard devices with default values: DMA controller (8237); Programmable Interrupt Controller (8259); Programmable Interval Timer (8254); RTC chip. |  |  |  |
| C1 | OEM Specific-Test to Size On-Board Memory |  |  | Initialize POST Error Manager (PEM) |
| C2 |  |  |  | Initialize error logging |
| CODE | Award |  | AMI | Phoenix4.0/Tandy3000 |
| C3 | Test the first 256K DRAM |  |  | Initialize error display function |
|  | Expand the compressed codes into temporary DRAM area including the compressed system BIOS & Option ROMs. |  |  |  |
| C4 |  |  |  | Initialize system error handler |
| C5 | OEM Specific-Early Shadow Enable for Fast Boot |  |  | PnPnd dual CMOS (optional) |
| C6 | External Cache Size Detection |  |  | Initialize note dock (optional) |
| C7 |  |  |  | Initialize note dock late |
| C8 |  |  |  | # Force check (optional) |
| C9 |  |  |  | Extended checksum (optional) |
| CA |  |  |  | Redirect Int 15h to enable remote keyboard |
| CB |  |  |  | Redirect Int 13h to Memory Technologies Devices such as ROM, RAM, PCMCIA, and serial disk |
| CC |  |  |  | Redirect Int 10h to enable remote serial video |
| CD |  |  |  | Re-map I/O and memory for PCMCIA |
| CE |  |  |  | Initialize digitizer and display message |
| D0 |  |  | The NMI is disabled. Power on delay is starting. Next, the initialization code checksum will be verified. |  |
| D1 |  |  | Initializing the DMA controller, performing the keyboard controller BAT test, starting memory refresh, and entering 4 GB flat mode next. |  |
| D2 |  |  |  | Unknown interrupt |
| CODE | Award |  | AMI | Phoenix4.0/Tandy3000 |
| D3 |  |  | Starting memory sizing next |  |
| D4 |  |  | Returning to real mode. Executing any OEM patches and setting the stack next. |  |
| D5 |  |  | Passing control to the uncompressed code in shadow RAM at E000:0000h. The initialization code is copied to segment 0 and control will be transferred to segment 0 |  |
| D6 |  |  | Control is in segment 0. Next, checking if <Ctrl> <Home> was pressed and verifying the system BIOS checksum. If either <Ctrl> <Home> was pressed or the system BIOS checksum is bad, next will go to checkpoint code E0h. Otherwise, going to checkpoint code D7h. |  |
| E0 |  |  | The onboard floppy controller if available is initialized. Next, beginning the base 512 KB memory test | Initialize the chipset |
| E1 | E1 Setup - Page E1 |  | Initializing the interrupt vector table next | Initialize the bridge |
| E2 | E2 Setup - Page E2 |  | Initializing the DMA and Interrupt controllers next. | Initialize the CPU |
| E3 | E3 Setup - Page E3 |  |  | Initialize system timer |
| E4 | E4 Setup - Page E4 |  |  | Initialize system I/O |
| E5 | E5 Setup - Page E5 |  |  | Check force recovery boot |
| CODE | Award |  | AMI | Phoenix4.0/Tandy3000 |
| E6 | E6 Setup - Page E6 |  | Enabling the floppy drive controller and Timer IRQs. Enabling internal cache memory. | Checksum BIOS ROM |
| E7 | E7 Setup - Page E7 |  |  | Go to BIOS |
| E8 | E8 Setup - Page E8 |  |  | Set Huge Segment |
| E9 | E9 Setup - Page E9 |  |  | Initialize Multi Processor |
| EA | EA Setup - Page EA |  |  | Initialize OEM special code |
| EB | EB Setup - Page EB |  |  | Initialize PIC and DMA |
| EC | EC Setup - Page EC |  |  | Initialize Memory type |
| ED | ED Setup - Page ED |  | Initializing the floppy drive. | Initialize Memory size |
| EE | EE Setup - Page EE |  | Looking for a floppy diskette in drive A:. Reading the first sector of the diskette | Shadow Boot Block |
| EF | EF Setup - Page EF |  | A read error occurred while reading the floppy drive in drive A:. | System memory test |
| F0 |  |  | Next, searching for the AMIBOOT.ROM file in the root directory. | Initialize interrupt vectors |
| F1 |  |  | The AMIBOOT.ROM file is not in the root directory | Initialize Run Time Clock |
| F2 |  |  | Next, reading and analyzing the floppy diskette FAT to find the clusters occupied by the AMIBOOT.ROM file | Initialize video |
| F3 |  |  | Next, reading the AMIBOOT.ROM file, cluster by cluster. | Initialize System Management Manager |
| F4 |  |  | The AMIBOOT.ROM file is not the correct size | Output one beep |
| F5 |  |  | Next, disabling internal cache memory. | Clear Huge Segment |
| F6 |  |  |  | Boot to Mini DOS |
| F7 |  |  |  | Boot to Full DOS |
| CODE | Award |  | AMI | Phoenix4.0/Tandy3000 |
| FB |  |  | Next, detecting the type of flash ROM. |  |
| FC |  |  | Next, erasing the flash ROM. |  |
| FD |  |  | Next, programming the flash ROM |  |
| FF | Int 19 Boot Attempt |  | Flash ROM programming was successful. Next, restarting the system BIOS. |  |

Ⅷ、Description of beep code

⑴AMI BIOS beep codes (fatal error)

| 1 beep | DRAM Refresh Failure. Try reseating the memory first. If the error still occurs, replace the memory with known good chips. |
| --- | --- |
| 2 beeps | Parity Error in First 64K RAM. Try reseating the memory first. If the error still occurs, replace the memory with known good chips. |
| 3 beeps | Base 64K RAM Failure. Try reseating the memory first. If the error still occurs, replace the memory with known good chips. |
| 4 beeps | System timer failure |
| 5 beeps | Process failure |
| 6 beeps | Keyboard Controller 8042 - Gate A20 Error. try reseating the keyboard controller chip. If the error still occurs, replace the keyboard chip. If the error persists, check parts of the system relating to the keyboard, e.g. try another keyboard, check to see if the system has a keyboard fuse |
| 7 beeps | Processor Virtual Mode Exception Interrupt Error |
| 8 beeps | Display Memory Read/Write Test Failure (Non-fatal). Replace the video card or the memory on the video card. |
| 9 beeps | ROM BIOS Checksum (32KB at F800:0) Failed. It is not likely that this error can be corrected by reseating the chips. Consult the motherboard supplier or an AMI product distributor for replacement part(s). |
| 10 beeps | CMOS Shutdown Register Read/Write Error |
| 11 beeps | Cache memory error |

⑵. AMI BIOS beep codes (Non-fatal error)

| 2 short | POST Failure - One or more of the hardware tests has failed |
| --- | --- |
| 1 long 2 short | An error was encountered in the video BIOS ROM, or a horizontal retrace failure has been encountered |
| 1 long 3 short | Conventional/Extended memory failure |
| 1 long 8 short | Display/Retrace test failed |

⑶. Award BIOS beep codes

| 1 short | No error during POST |
| --- | --- |
| 2 short | Any Non-fatal error, enter CMOS SETUP to reset |
| 1 long 1 short | RAM or motherboard error |
| 1 long 2 short | Video Error, Cannot Initialize Screen to Display Any Information |
| 1 long 3 short | Keyboard Controller error |
| 1 long 9 short | Flash RAM/EPROM (which on the motherboard) error. (BIOS error) |
| Long beep | Memory bank is not plugged well, or broken. |

⑷.Phoenix BIOS beep codes

| Beep Code | ## Description / What to Check |
| --- | --- |
| 1-1-1-3 | Verify Real Mode. |
| 1-1-2-1 | Get CPU type. |
| 1-1-2-3 | Initialize system hardware. |
| 1-1-3-1 | Initialize chipset registers with initial POST values. |
| 1-1-3-2 | Set in POST flag. |
| 1-1-3-3 | Initialize CPU registers. |
| 1-1-4-1 | Initialize cache to initial POST values. |
| 1-1-4-3 | Initialize I/O. |
| 1-2-1-1 | Initialize Power Management. |
| 1-2-1-2 | Load alternate registers with initial POST values. |
| 1-2-1-3 | Jump to UserPatch0. |
| 1-2-2-1 | Initialize keyboard controller. |
| 1-2-2-3 | BIOS ROM checksum. |
| 1-2-3-1 | 8254 timer initialization. |
| 1-2-3-3 | 8237 DMA controller initialization. |
| 1-2-4-1 | Reset Programmable Interrupt Controller. |
| 1-3-1-1 | Test DRAM refresh. |
| 1-3-1-3 | Test 8742 Keyboard Controller. |
| 1-3-2-1 | Set ES segment to register to 4 GB. |
| 1-3-3-1 | 28 Autosize DRAM. |
| 1-3-3-3 | Clear 512K base RAM. |
| 1-3-4-1 | Test 512K base address lines. |
| 1-3-4-3 | Test 512K base memory. |
| 1-4-1-3 | Test CPU bus-clock frequency. |
| 1-4-2-4 | Reinitialize the chipset. |
| 1-4-3-1 | Shadow system BIOS ROM. |
| 1-4-3-2 | Reinitialize the cache. |
| 1-4-3-3 | Auto size cache. |
| 1-4-4-1 | Configure advanced chipset registers. |
| 1-4-4-2 | Load alternate registers with CMOS values. |
| 2-1-1-1 | Set Initial CPU speed. |
| 2-1-1-3 | Initialize interrupt vectors. |
| 2-1-2-1 | Initialize BIOS interrupts. |
| 2-1-2-3 | Check ROM copyright notice. |
| 2-1-2-4 | Initialize manager for PCI Options ROMs. |
| 2-1-3-1 | Check video configuration against CMOS. |
| 2-1-3-2 | Initialize PCI bus and devices. |
| 2-1-3-3 | Initialize all video adapters in system. |
| 2-1-4-1 | Shadow video BIOS ROM. |
| 2-1-4-3 | Display copyright notice. |
| 2-2-1-1 | Display CPU type and speed. |
| 2-2-1-3 | Test keyboard. |
| 2-2-2-1 | Set key click if enabled. |
| 2-2-2-3 | 56 Enable keyboard. |
| 2-2-3-1 | Test for unexpected interrupts. |
| 2-2-3-3 | Display prompt "Press F2 to enter SETUP". |
| 2-2-4-1 | Test RAM between 512 and 640k. |
| 2-3-1-1 | Test expanded memory. |
| 2-3-1-3 | Test extended memory address lines. |
| 2-3-2-1 | Jump to UserPatch1. |
| 2-3-2-3 | Configure advanced cache registers. |
| 2-3-3-1 | Enable external and CPU caches. |
| 2-3-3-3 | Display external cache size. |
| 2-3-4-1 | Display shadow message. |
| 2-3-4-3 | Display non-disposable segments. |
| 2-4-1-1 | Display error messages. |
| 2-4-1-3 | Check for configuration errors. |
| 2-4-2-1 | Test real-time clock. |
| 2-4-2-3 | Check for keyboard errors |
| 2-4-4-1 | Set up hardware interrupts vectors. |
| 2-4-4-3 | Test coprocessor if present. |
| 3-1-1-1 | Disable onboard I/O ports. |
| 3-1-1-3 | Detect and install external RS232 ports. |
| 3-1-2-1 | Detect and install external parallel ports. |
| 3-1-2-3 | Re-initialize onboard I/O ports. |
| 3-1-3-1 | Initialize BIOS Data Area. |
| 3-1-3-3 | Initialize Extended BIOS Data Area. |
| 3-1-4-1 | Initialize floppy controller. |
| 3-2-1-1 | Initialize hard-disk controller. |
| 3-2-1-2 | Initialize local-bus hard-disk controller. |
| 3-2-1-3 | Jump to UserPatch2. |
| 3-2-2-1 | Disable A20 address line. |
| 3-2-2-3 | Clear huge ES segment register. |
| 3-2-3-1 | Search for option ROMs. |
| 3-2-3-3 | Shadow option ROMs. |
| 3-2-4-1 | Set up Power Management. |
| 3-2-4-3 | Enable hardware interrupts. |
| 3-3-1-1 | Set time of day. |
| 3-3-1-3 | Check key lock. |
| 3-3-3-1 | Erase F2 prompt. |
| 3-3-3-3 | Scan for F2 key stroke. |
| 3-3-4-1 | Enter SETUP. |
| 3-3-4-3 | Clear in-POST flag. |
| 3-4-1-1 | Check for errors |
| 3-4-1-3 | POST done--prepare to boot operating system. |
| 3-4-2-1 | One beep. |
| 3-4-2-3 | Check password (optional). |
| 3-4-3-1 | Clear global descriptor table. |
| 3-4-4-1 | Clear parity checkers. |
| 3-4-4-3 | Clear screen (optional). |
| 3-4-4-4 | Check virus and backup reminders. |
| 4-1-1-1 | Try to boot with INT 19. |
| 4-2-1-1 | Interrupt handler error. |
| 4-2-1-3 | Unknown interrupt error. |
| 4-2-2-1 | Pending interrupt error. |
| 4-2-2-3 | Initialize option ROM error. |
| 4-2-3-1 | Shutdown error. |
| 4-2-3-3 | Extended Block Move. |
| 4-2-4-1 | Shutdown 10 error. |
| 4-3-1-3 | Initialize the chipset. |
| 4-3-1-4 | Initialize refresh counter. |
| 4-3-2-1 | Check for Forced Flash. |
| 4-3-2-2 | Check HW status of ROM. |
| 4-3-2-3 | BIOS ROM is OK. |
| 4-3-2-4 | Do a complete RAM test. |
| 4-3-3-1 | Do OEM initialization. |
| 4-3-3-2 | Initialize interrupt controller. |
| 4-3-3-3 | Read in bootstrap code. |
| 4-3-3-4 | Initialize all vectors. |
| 4-3-4-1 | Boot the Flash program. |
| 4-3-4-2 | Initialize the boot device. |
| 4-3-4-3 | Boot code was read OK. |

⑸．IBM BIOS beep codes

| Beep Code | Description |
| --- | --- |
| No Beeps | No Power, Loose Card, or Short. |
| 1 Short Beep | Normal POST, computer is ok. |
| 2 Short Beep | POST error, review screen for error code. |
| Continuous Beep | No Power, Loose Card, or Short. |
| Repeating Short Beep | No Power, Loose Card, or Short. |
| One Long and one Short Beep | Motherboard issue. |
| One Long and Two short Beeps | Video (Mono/CGA Display Circuitry) issue. |
| One Long and Three Short Beeps. | Video (EGA) Display Circuitry. |
| Three Long Beeps | Keyboard / Keyboard card error. |
| One Beep, Blank or Incorrect Display | Video Display Circuitry. |

Ⅸ、 Frequently-asked questions

| situations | causes | solutions |
| --- | --- | --- |
| The indicator lights, but the number LED is not lit; Number LED lights 1 bit, 2 bits or 3 bits | The card may be loose | Power off, remove the card and clean the pin with eraser and try again. |
|  | The ISA slot may be dirty | Clean the dirt in the slot, try to insert and remove the card frequently, so as to wipe out the dirt in the slot. |
|  | the interval of restart is too short | Restart the computer only 8 second after you power off it. |
| The function switch doesn't work | Your hands may touch the pins of function switch and result in faradism that make the keyboard doesn't work. | Stick a insulating tape to the switch pin. |
|  | While you press the switch the card may be loose | Hold the switch for proper time: press and hold the switch for proper time and then leave go of as soon as the POST card respond. |
| The function switch works, but it doesn't operate well | The time you hold the switch is incorrect (0.8 second is the longer one, 0.4 second is the shorter one.) | Press the switch as gently as you can to make sure he card inserted well in the slot. |
|  | The switch is bad | contact your dealer to change the switch |